Onderzoeksopdracht Maritieme Techniek

| Toegewezen aan: ###### Nr #### 01-025 #### SOC 1 | Sectie opdrachtgever Ship and Offshore Structures |  | Naam opdrachtgever Ir. J.H. Vink |  |  | Datum March 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Titel | ## Collapse load of stiffened plate elements |  |  |  |  |  |
| Trefwoorden |  |  |  |  |  |  |
| Inleiding | It is becoming more and more practice to evaluate the ultimate strength of the ship hull and its components, in order to get better insight in the safety factor with regard to collapse due to extreme loads. The ultimate longitudinal strength of the hull girder is evaluated with help of so called “Load shortening curves” of individual stiffened panels in the midship section. In literature several analytical and (semi)-empirical methods are described to predict these “Load shortening curves” of the plate-stiffener combinations. Besides that it is also possible to predict the collapse behavior of these elements with help of non-linear FEM models with built in imperfections. This research is intended to study and apply the different methods available for predicting the “Load shortening curve” for a given configuration, and to compare them. An experimental verification has to be prepared and executed as well, where special attention has to be paid to realizing proper boundary conditions. |  |  |  |  |  |
| Vraagstelling |  |  |  |  |  |  |
| Uitwerking en eisen | Contents: - Introduction with a clear statement of the problem, - Results of literature search, - Theoretical methods: possibilities, restrictions, predicted results - Semi empirical methods: assumptions, simplifications, predicted results - Non-linear FEM analysis: mesh, boundary conditions, influence of assumed imperfections, predicted results - Comparison of the results from these prediction methods, - Design of experimentel setup, - Experimental results, - Conclusions and recommendations. Basic knowledge: Colleges: Applied mechanics 1 and 2 Colleges: Structural design and analysis of marine objects 1, 2 and 3 Colleges: FEM |  |  |  |  |  |
| Opmerkingen | Literature: - Ultimate longitudinal strength, B.J. Caldwell. Transactions RINA, 1965, pp 411 - Evaluation of ultimate ship hull strength, R.S. Dow, R.C. Hugill, J.D. Clark, G.C. Smith. Extreme loads Response Symposium, Arlington, Oct 1981, pp 133 - Ultimate longitudinal strength of ships: A case study, S.E. Rutherford, J.B. Caldwell. SNAME Transactions Vol 98, 1990, pp 441 - Methods for the assessment of structural strength of naval ships, E. Nieszen, P.J. Keuning. T.N. Bosman, L.F. Galle. Proceedings 5th IMDC, TUDelft, 1994, pp 77 - Progressive collapse behaviour of hull girder of a bulkcarrier under longitudinal bending, T. Yao, P.I. Nikolov. NK Technical bulletin, 1993, pp 5 - 12th ISSC report 1995, Report of Committee III.1: Ductile collapse - 12th ISSC report 1995, Report of Committee V.1: Strength limit states formulations - Ultimate and crushing strength of plated structures J.K. Paik, P.T. Pedersen. Journal of ship research Vol 39, no 3, Sept 1995, pp 250 - A simplified method for predicting ultimate compressive strength of ship panels, J.K. Paik, P.T. Pedersen, ISP no 434, 1996, pp139 - Estimation of ultimate longitudinal bending moment of ships and box girders, M.K. Rahman, M. Chowdhury, Journal of ship research, Vol 40, No 3, Sept 1996, pp 244 - Approximate assessment of the ultimate longitudinal strength of the hull girder, J.M. Gordo, C. Guedes Soares, D. Faulkner. Journal of ship research, Vol 40, no 1, March 1996, pp 60 |  |  |  |  |  |
| Begeleiding | ###### Naam | Sectie |  | Tel | E-mail |  |
|  | Ir. J.H. Vink | SOC |  | 85923 | j.h.vink@wbmt.tudelft.nl |  |